|  |  | ## RV readiness checklist How To Full time RV.com # RV Create A Maintenance Plan (CAMP) When’s the right time to do your road trip safety checklist? Every time you plan to hit the road. This is the recommended maintenance that you need perform on your RV Refrigerator to keep it functioning properly. |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Refrigerator Overview for Start Up - Make sure inspect the refrigerator Inside the fridge and the freezer - Clean any dust or mildew from both areas - Plug in the fridge and let it run for at least 12 hours before you fill it - Look into the proper way to fill a RV refrigerator so air flows - Get a batter- operated air fan for your RV fridge so air flows Maintain Inside - Clean any mildew, dirt, or grime from the door seals- this will help the fridge run better - Check the temperature- this could indicate a possible problem - This is operation but keep the refrigerator filled when in use Outside - Remove the access on the side of the RV, take a vacuum and clean any condenser coils that you can get to- dirt can affect the proper function of fridge and can reduce its life. - You can use a coil cleaner for you coils on the refrigerator, heating unit, and air conditioner, called QwikProducts Foaming Coil Cleaner. - Look for rust on the flue- this could be an indication of moisture in the burner area. - Now go on the roof- remove the access cover above the refrigerator, clean any debris or leaves from the screen. Inspect the screen that protects debris from damaging the fridge, if it is damaged, replace it. Make sure that nothing is preventing proper circulation. Maintenance should be performed on the refrigerator after each trip in the RV. - Check that all wires and plugs are still connected properly - Check for leaves and debris, especially around the refrigerator’s exhaust. - When possible, park the RV so the side that the refrigerator is on, is in the shade and the RV is level. - After using the RV for a weekend trip or even Full Time, clean the inside of the refrigerator with a damp cloth, defrost the fins in the freezer, and clean the seals. Cost - QwikProducts Foaming Coil Cleaner $12 - An RV refrigerator often costs thousands more than a home one does. Then again, a home refrigerator does not work on both propane and electricity. The Norcold in my Newmar Class A retails for $4,000. |  |  |  |  |  |